TEMASEK POLYTECHNIC SCHOOL OF INFORMATICS & IT

Diploma in Cybersecurity and Digital Forensics

AY 2022/2023 PROJECT REQUIREMENTS

(Oct Semester)

NETWORK SECURITY (CCF2C01)

SUBJECT LEVEL: 2

TIME ALLOWED: DUE ON WEEK 7

INSTRUCTIONS TO CANDIDATES

- This paper consists of 9 pages (including the cover page).

- This assignment constitutes 50% of the total marks for this subject.

Project Deliverables (50%, Total 100 Marks)

This project is to be done in a group of 2 (maximum) students.

- Packet Tracer Solution (Individual - 20 Marks)

  - Part 1 - Team Member #1

  - Part 2 - Team Member #2

- Report

  - Documentation (Group - 20 Marks)

- Research (Individual - 30 Marks)

- Q&A in week 7 during the 2nd Second Session (Individual - 30 Marks)

- Please refer to the MAPS framework when ding your project.

| MAPS Framework Infographic | MAPS Framework PDF |  |
| --- | --- | --- |
| MAPS Framework Video | MAPS Framework Video |  |

Assignment report (softcopy) and Packet Tracer implementation submission deadline via LMS:

Week 7 (before 2nd Session 2359Hrs)

Task 1: Overview and Objectives

This project requires your team to:

| Done by | Tasks |
| --- | --- |
| Member #1 | Part 1: Configure an ASA firewall to implement security policies. |
|  | Part 2: Configure Layer 2 security on a LAN switch. |
| Member #2 |  |
|  | Part 3: Configure a site-to-site IPsec VPN |

Scenario

Your team has been hired by a Tee Phee (TP) Pte Ltd. that has a corporate headquarters and multiple branch offices. TP wants your team to implement the network according to Figure 1. The TP is also concerned about network security and wants you to implement Layer 2 security, an ASA device, and VPN services from HQ to the branches according to Figure 1. Your job is to prototype the network in the lab prior to your company installing the equipment at the TP sites. In this project, you will only implement a VPN between headquarters and a single branch (as shown in Figure 1).

Refer to Appendix A for IP addresses as assigned by your tutor.

IP Address Requirements

HQ DMZ Network	: 192.168.12.0/24

HQ Internal Network	: 192.168.32.0/24

HQ Outside Network	: 209.165.201.240/28

HQ BRANCH Network	: 199.133.201.32/27

DMZ Web Server Public IP:   209.165.X.241    DMZ DNS Server Public IP:   209.165.X.242    HQ-internet WAN	: 209.165.201.224/30

Branch-Internet WAN	: 198.133.201.0/30

External-internet WAN	: 192.31.2.0/30

Figure 1

# Part 1: Configure the ASA 5506-X

## Step 1: Configure Basic Settings on the ASA device.

Set HQ-ASA5506 with a password: C1sc0.

- Configure the domain name as TP.com.

- Configure the hostname as HQ-ASA5506.

- Configure the INSIDE, OUTSIDE, and DMZ interfaces with the following:

  - nameif OUTSIDE, security-level 1, assign to G1/1

  - nameif INSIDE, security-level 100, assign to G1/2

  - nameif DMZ, security-level 70, assign to G1/3

## Step 2: Configure the DHCP service on the ASA device for the internal network.

- Create DHCP Pool according to IP address provided by your tutor.

- The DHCP service should provide DNS server (AAA/NTP/syslog server) information.

- PC0, PC1, and PC4 should receive their addresses over DHCP.

## Step 3: Configure routing on the ASA.

Configure a default route that will enable hosts on the HQ INTERNAL and DMZ networks to communicate with outside hosts. Use the IP address of the HQ router interface as the gateway interface.

## Step 4: Configure Secure Network Management for the ASA Device.

- Configure the ASA with NTP and AAA:

  - The ASA is a NTP client to the AAA/NTP/Syslog server.

  - Enable the authentication to the ASA.

  - The authentication key is key 1 with the password is corpkey.

- Configure AAA and SSH.

  - Configure the ASA device with AAA authentication using the username of TPAdmin and password of adminpass01.

  - Configure AAA to use the local database for SSH connections to the console port.

  - Configure HQ-ASA5506 to accept SSH connections only from the Net Admin workstation.

  - Configure SSH session timeout to be 20 minutes.

## Step 5: Configure NAT Service for the ASA device for both INSIDE and DMZ networks.

- Create a network object called INSIDE-nat with the given subnet 192.168.x.0/24 and enable the IP addresses of the hosts in the internal network to be dynamically translated to access the external network via the outside interface.

- Create a network object DMZ-web-server to statically translate the DMZ web server internal IP address to the outside public IP address 209.165.X.241.

- Create a network object DMZ-dns-server to statically translate the DMZ DNS server internal IP address to the outside public IP address 209.165.X.242.

## Step 6: Configure ACL on the ASA device to implement the Security Policy.

- Configure a named extended ACL to permit inside hosts to be translated to the pool of outside IP addresses. Name the ACL NAT-IP-ALL.

- Apply NAT-IP-ALL ACL to the DMZ and OUTSIDE interfaces in the inward direction.

- Configure an ACL to allow access to the DMZ servers from the internet. Create an extended named ACL (named OUTSIDE-TO-DMZ) to filter incoming traffic to the HQ ASA. The ACL statements should be created in the order specified in the following guidelines:

  - The ACL should contain four access control entries (ACEs).

  - HTTP traffic is allowed to DMZ Web Server.

  - DNS traffic (both TCP and UDP) is allowed to the DMZ DNS server (two separate ACEs).

  - FTP traffic from the Branch administrator workstation is allowed to the DMZ web server.

# Part 2: Configure Layer 2 Security on HQ-Switch1

Configure HQ-Switch1 in the internal network with Layer 2 attack mitigation measures.

## Step 1: Disable Unused Switch Ports Step 2: Implement Port Security

On HQ-Switch1, configure port security on all of the switch ports that are connected to hosts according to the following requirements:

- The ports should be configured as static access ports.

- The ports should learn a maximum of two MAC addresses.

- The ports should record the MAC addresses that have been learned in the device running configuration.

- If a violation occurs, the port should drop packets from host MAC addresses that have not been learned, increment the violation counter, and generate a syslog message.

## Step 3: Implement STP Security

On HQSwitch1, implement STP security measures on the active ports that are connected to hosts.

- Configure the switch to disable host ports that receive a BPDU.

- Configure the ports to quickly go into STP forwarding mode without going through the STP transitional modes.

# Part 3: Configure a Site-to-Site IPsec VPN between the HQ and the Branch Routers

Configure the Branch and HQ routers with a username of CORPADMIN and a password of

NetSec-Admin1. The enable secret password should be RTR-AdminP@55.

Configure a site-to-site IPsec VPN between the HQ and Branch routers according to the requirements below.

The following tables list the parameters for the ISAKMP phase 1 and phase 2 policies:

### ISAKMP Phase 1 Policy Parameters

| Key Distribution Method | ISAKMP |
| --- | --- |
| Encryption Algorithm | AES |
| Number of Bits | 256 |
| Hash Algorithm | SHA-1 |
| Authentication Method | Pre-share |
| Key Exchange | DH 2 |
| IKE SA Lifetime | 1800 |
| ISAKMP Key | Vpnpass101 |

IPsec Phase 2 Policy Parameters Table

| Parameters | HQ Router | Branch Router |
| --- | --- | --- |
| Transform Set Name | VPN-SET | VPN-SET |
| Transform Set | esp-aes esp-sha-hmac | esp-aes esp-sha-hmac |
| Peer Host Name | Branch | HQ |
| Peer IP Address | 198.133.X.X | 209.165.X.X |
| Encrypted Network | 209.165.X.X/28 | 198.133.X.X/27 |
| Crypto Map Name | VPN-MAP | VPN-MAP |
| SA Establishment | ipsec-isakmp | ipsec-isakmp |

- Configure ACL 120 on the HQ router to identify the interesting traffic to be sent across the VPN. The interesting traffic is all IP traffic from the HQ LAN to the Branch LAN.

- Configure the ISAKMP Phase 1 properties on the HQ router. The crypto ISAKMP policy is 10. Refer to the ISAKMP Phase 1 Policy Parameters Table for the specific details needed.

- Configure the ISAKMP Phase 2 properties on the HQ router using 10 as the sequence number. Refer to the ISAKMP Phase 2 Policy Parameters Table for the specific details needed.

- Bind the VPN-MAP crypto map to the outgoing interface.

- Configure IPsec parameters on the Branch router using the same parameters as on the HQ router. Note that interesting traffic is defined as the IP traffic from the Branch LAN to the LAN that is attached to HQ.

- Save the running-config, then reload both the HQ and Branch routers.

Task 2: Report

Documentation (20 Marks)

Using the given report template, document the configurations done for Tasks 1 above.

Task 3: Research.

Do independent research (either from online, LMS e-learning materials, library resources, etc.) and write a (maximum) one-page summary (diagrams excluded) on ONE of the following cryptographic system concepts:

- CIA (Confidentiality, Integrity, Availability) and Non-repudiation

- AAA (Authentication, Authorisation, Accounting)

- Symmetric encryption systems

- Asymmetric encryption systems

- Hashing systems (and possibly HMAC or Hash Message Authentication Code)

- Key exchange systems (e.g., Diffie-Hellman algorithm)

- Digital signature and certification systems

- Any other cryptographic system topic (prior approval must be obtained from your lab tutor

before you submit your own topic in your report)

Note: The emphasis should be on the basic security concepts and mechanisms and not on a particular algorithm or implementation (e.g., the intent and purpose of a hashing system, instead of the particular MD5/SHA-1/SHA-256/SHA-512 hashing algorithm and its implementation).

### Marks allocation:

- Accuracy – 10 marks

- Clarity & conciseness – 10 marks

- Completeness – 10 marks

Task 4: Q&A (30 Marks)

The Q&A Session will be done during the project presentation. Students will be quizzed the theories and concepts on:

  - all topics covered in labs and project

  - packet tracer configurations

  - research report

*** THE END ***

Appendix A – IP address assignment to the project groups

| Group. | HQ DMZ LAN | HQ Internal LAN | HQ OUTSIDE LAN | HQ BRANCH LAN | Public LAN 1 | Public LAN 2 | HQ-Internet WAN | BRANCH- Internet WAN | EXTERNAL- Internet WAN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 192.168.11.0/24 | 192.168.31.0/24 | 209.165.200.240/28 | 198.133.200.32/27 | 192.135.250.0/24 | 192.31.7.32/27 | 209.165.200.224/30 | 198.133.200.0/30 | 192.31.1.0/30 |
| 2 | 192.168.12.0/24 | 192.168.32.0/24 | 209.165.201.240/28 | 198.133.201.32/27 | 192.135.250.0/24 | 192.31.7.32/27 | 209.165.201.224/30 | 198.133.201.0/30 | 192.31.2.0/30 |
| 3 | 192.168.13.0/24 | 192.168.33.0/24 | 209.165.202.240/28 | 198.133.202.32/27 | 192.135.250.0/24 | 192.31.7.32/27 | 209.165.202.224/30 | 198.133.202.0/30 | 192.31.3.0/30 |
| 4 | 192.168.14.0/24 | 192.168.34.0/24 | 209.165.203.240/28 | 198.133.203.32/27 | 192.135.250.0/24 | 192.31.7.32/27 | 209.165.203.224/30 | 198.133.203.0/30 | 192.31.4.0/30 |
| 5 | 192.168.15.0/24 | 192.168.35.0/24 | 209.165.204.240/28 | 198.133.204.32/27 | 192.135.250.0/24 | 192.31.7.32/27 | 209.165.204.224/30 | 198.133.204.0/30 | 192.31.5.0/30 |
| 6 | 192.168.16.0/24 | 192.168.36.0/24 | 209.165.205.240/28 | 198.133.205.32/27 | 192.135.250.0/24 | 192.31.7.32/27 | 209.165.205.224/30 | 198.133.205.0/30 | 192.31.6.0/30 |
| 7 | 192.168.17.0/24 | 192.168.37.0/24 | 209.165.206.240/28 | 198.133.206.32/27 | 192.135.250.0/24 | 192.31.7.32/27 | 209.165.206.224/30 | 198.133.206.0/30 | 192.31.7.0/30 |
| 8 | 192.168.18.0/24 | 192.168.38.0/24 | 209.165.207.240/28 | 198.133.207.32/27 | 192.135.250.0/24 | 192.31.7.32/27 | 209.165.207.224/30 | 198.133.207.0/30 | 192.31.8.0/30 |
| 9 | 192.168.19.0/24 | 192.168.39.0/24 | 209.165.208.240/28 | 198.133.208.32/27 | 192.135.250.0/24 | 192.31.7.32/27 | 209.165.208.224/30 | 198.133.208.0/30 | 192.31.9.0/30 |
| 10 | 192.168.20.0/24 | 192.168.40.0/24 | 209.165.209.240/28 | 198.133.209.32/27 | 192.135.250.0/24 | 192.31.7.32/27 | 209.165.209.224/30 | 198.133.209.0/30 | 192.31.10.0/30 |
| 11 | 192.168.21.0/24 | 192.168.41.0/24 | 209.165.210.240/28 | 198.133.210.32/27 | 192.135.250.0/24 | 192.31.7.32/27 | 209.165.210.224/30 | 198.133.210.0/30 | 192.31.11.0/30 |
| 12 | 192.168.22.0/24 | 192.168.42.0/24 | 209.165.211.240/28 | 198.133.211.32/27 | 192.135.250.0/24 | 192.31.7.32/27 | 209.165.211.224/30 | 198.133.211.0/30 | 192.31.12.0/30 |
| 13 | 192.168.23.0/24 | 192.168.43.0/24 | 209.165.212.240/28 | 198.133.212.32/27 | 192.135.250.0/24 | 192.31.7.32/27 | 209.165.212.224/30 | 198.133.212.0/30 | 192.31.13.0/30 |
| 14 | 192.168.24.0/24 | 192.168.44.0/24 | 209.165.213.240/28 | 198.133.213.32/27 | 192.135.250.0/24 | 192.31.7.32/27 | 209.165.213.224/30 | 198.133.213.0/30 | 192.31.14.0/30 |
| 15 | 192.168.25.0/24 | 192.168.45.0/24 | 209.165.214.240/28 | 198.133.214.32/27 | 192.135.250.0/24 | 192.31.7.32/27 | 209.165.214.224/30 | 198.133.214.0/30 | 192.31.15.0/30 |
| 16 | 192.168.26.0/24 | 192.168.46.0/24 | 209.165.215.240/28 | 198.133.215.32/27 | 192.135.250.0/24 | 192.31.7.32/27 | 209.165.215.224/30 | 198.133.215.0/30 | 192.31.16.0/30 |
| 17 | 192.168.27.0/24 | 192.168.47.0/24 | 209.165.216.240/28 | 198.133.216.32/27 | 192.135.250.0/24 | 192.31.7.32/27 | 209.165.216.224/30 | 198.133.216.0/30 | 192.31.17.0/30 |
| 18 | 192.168.28.0/24 | 192.168.48.0/24 | 209.165.217.240/28 | 198.133.217.32/27 | 192.135.250.0/24 | 192.31.7.32/27 | 209.165.217.224/30 | 198.133.219.0/30 | 192.31.18.0/30 |